The user is asking about the derivation of the value β₁L = 22.3733 used in the vibrating beam equation for Young's modulus calculation. This is a classical mechanics problem related to the natural frequencies of a free-free beam (or cantilever beam).

Let me think about this carefully.

For a beam with free-free boundary conditions, the natural frequencies are determined by the equation for transverse vibration of a beam. The equation of motion for a free-free beam involves solving the Euler-Bernoulli beam equation.

The characteristic equation for a free-free beam is:

cos(βL) · cosh(βL) = 1

For the first mode (n=1) of a free-free beam, the solution to this transcendental equation gives β₁L ≈ 4.7300 (this is for the first flexible mode, as the first two modes are rigid body modes).

Actually, I need to reconsider the boundary conditions here. Different beam configurations have different characteristic equations—a cantilever beam gives β₁L ≈ 1.8751, a free-free beam gives β₁L ≈ 4.7300 for the first flexible mode, and a simply supported beam follows βL = nπ. The value 22.3733 doesn't match any of these standard cases, so I'm wondering if this might be related to a higher mode or a different boundary condition altogether. ≈ 3.1416

Now, 22.3733... let me think. If we square some of these values:

• 4.7300² ≈ 22.37

Yes! β₁L = 4.7300 for the first free mode of a free-free beam, and (β₁L)² = 4.7300² ≈ 22.3733!

So the 22.3733 is actually (β₁L)² where β₁L ≈ 4.7300 is the first eigenvalue for a free-free beam.

The equation E = [48 * π² * ρ * f² * L⁴] / [ (β₁L)² * t² ] - wait, but that doesn't quite make sense with the units if (β₁L) is already squared.

Actually, let me reconsider the standard equation. For a free-free beam, the natural frequency formula involves (β_nL)² divided by L² and multiplied by the stiffness-to-mass ratio. For a rectangular cross-section, I need to express the moment of inertia and area in terms of the beam dimensions.

Now I'm simplifying the relationship between the measured frequency and Young's modulus by substituting the geometric properties and the eigenvalue for the first mode.

I'm noticing a discrepancy—my derivation gives (β₁L)⁴ in the denominator, but the user's equation has (β₁L)² multiplied by t². Let me reconsider whether these might actually be equivalent or if there's a different form being used.

The 22.3733 value is actually the square of 4.7300, which is the first eigenvalue β₁L for a free-free beam satisfying the transcendental equation cos(βL)·cosh(βL) = 1. Now I'm deriving the formula for Young's modulus from the frequency equation. I'll express the frequency in terms of the beam parameters, then rearrange to solve for E in terms of the measured frequency, material density, and beam dimensions.

I'm verifying the calculation of (β₁L)⁴ — since β₁L ≈ 4.7300, squaring this gives approximately 22.3733, and squaring again yields around 500.56, which checks out for the denominator.

The key insight is that 22.3733 might be presented in the luthiery literature as a pre-computed constant rather than the raw eigenvalue, which would explain why it appears directly in some formulations of the equation. So the constant 22.3733 represents (β₁L)², where β₁L ≈ 4.7300 is the first eigenvalue for a free-free beam—this is the characteristic frequency parameter that appears in the denominator of the Young's modulus equation. The key insight is recognizing whether the equation uses (22.3733 * t)² in the denominator, which would correctly account for the (β₁L)⁴ term needed in the formula.

This eigenvalue comes directly from solving the transcendental equation cos(βL) · cosh(βL) = 1, which emerges when applying the free-free boundary conditions (zero bending moment and zero shear force at both ends) to the Euler-Bernoulli beam vibration equation. So 22.3733 isn't an empirical constant at all—it's a pure mathematical result derived from the physics of a beam with free ends.