Key Metrics from Tap Testing

1. Speed of Sound (Longitudinal & Cross-grain)

By measuring the resonant frequency of a rectangular plate with known dimensions:

c = 2 × L × f₁

Where L is length and f₁ is the fundamental frequency along that axis.

2. Young's Modulus (Stiffness)

The most important metric for tonewood:

E = ρ × c²

Where ρ is density (mass/volume) and c is speed of sound.

You can measure E_long (along grain) and E_cross (across grain) separately.

3. Stiffness-to-Weight Ratio (Specific Modulus)

This is arguably the most critical metric for soundboards:

E/ρ = c²

Higher values mean the wood can vibrate more freely - it's stiff but light.

Typical values for good spruce:

• E_long/ρ: 20-28 GPa/(g/cm³)

• E_cross/ρ: 0.8-1.5 GPa/(g/cm³)

4. Radiation Ratio (Sound Radiation Coefficient)

Predicts how efficiently the wood converts vibration to sound:

R = √E / ρ  or  R = c / ρ

5. Cross-grain to Long-grain Ratio

The ratio E_cross/E_long (typically 0.04-0.08 for spruce) affects how the top responds across different modes.